USB Network Link Cable

Congratulations on your purchase of the USB-USB Network Bridge cable. This unique cable allows you to easily construct a network via USB port and share all the resource such as files, printers peripheral equipment, and more.

Please follow the steps to install the driver in your computer.

- Connection

Network Construction :

- Computer A : Connects Computer B with USB-USB Network Bridge Cable.

- Computer B : The Bridge Computer which connects computer A through

USB- USB Network Bridge Cable and connects Computer C with an Ethernet Adapter.

-Computer C : One PC of the Ethernet LAN, connects Computer B with an Ethernet Adapter and can access the internet directly.

Driver Installation :

Note : Please install driver in Computer A & Computer B.

Before you start installing the driver, please don’t plug your USB-USB Network Bridge Cable into the USB port of your computer. Follow the step below to install the USB-USB Network Bridge driver.

- Power on the computers where you will connect USB-USB Network Bridge Cable and make sure that the USB port is enabled and working properly.

- Insert the USB-USB Network Bridge driver diskette into the floppy disk drive and run the USB-USB Network Bridge 1.6.0.0 program inside the driver diskette.

- The install Shield Wizard will guide you step-by-step to complete the installation process.

- When the program is running, it will show as follows:

- Click “Next” To Continue ~

- Select the folder

Click “Next” To Continue ~

- If you are installing the driver in Computer A, Please click “No”.

- If you are installing the driver in Computer B, Please click “Yes”.

- You do not have to setup the IP Address, so please click “No”

※ Please restart the computer.

- Please install the driver in another computer A/B.

- Note: Please setup 2 PCs in the same “Workgroup” and Different Host Name.

After finished the steps as above.

Please check the “Network Neighborhood”.

You will see each other in Computer A and B.

That means you are connecting with “Intranet”.

- If you will connect with Internet. Please follow the steps.

- Before setup, please make sure Internet of “Computer B” is available.

- Please setup up Computer A&B as follows:

- Move your cursor to “Network Neighborhood” and click right button.

- Go to “Properties” and then “Configuration”.

- Choose “TCP/IP->USB-USB Network Bridge Adapter”. Click “Properties”.

- Please choose “DNS Configuration”. Enable DNS.

- Please setup your “Host Name” and Domain (Workgroup).

- Enter your DNS server.

- Restart your computer.

- Your computer A will connect with Internet.